# 1. Purpose

Static timing analysis (STA) checks whether every signal path in a chip can finish within one clock cycle, without simulating any input patterns. It adds up the delay of every gate along each path and compares the total with the time the clock allows.

This document covers STA of the vsdbabysoc gate-level netlist, run with OpenSTA. The same design and the same constraints were analysed 16 times, once for each process/voltage/temperature (PVT) corner, so any difference between the results is caused only by the operating conditions. For each corner the run reports the single worst setup path against an 11 ns clock.

Sections 2 to 4 explain the inputs and how to read a report. Section 5 explains the timing summary screenshots and charts. Sections 6 to 8 give the results, and Section 9 contains the full OpenSTA output for every corner.

# 2. Files used

| File | What it does |
| --- | --- |
| vsdbabysoc.synth.v | Gate-level netlist from synthesis. It is the circuit being analysed, and every instance in the path reports (core/U474, core/U9, and so on) comes from it. |
| sdc/vsdbabysoc_synthesis.sdc | Timing constraints. It defines the clock named clk (the reports show an 11 ns period) and the other rules the design must meet. Without it STA has no target to compare the paths against. |
| OpenSTA script | Runs the analysis once per corner, prints a "Running STA: <corner>" banner and the worst path for each, then prints the timing summary table followed by the report_worst_slack and report_tns results. |

# 3. Corner names

A name such as SS_n40C_1v28 has three parts: the process (SS), the temperature (n40C means -40 °C) and the supply voltage (1v28 means 1.28 V).

- FF (fast-fast): faster than typical transistors. This is the optimistic case for setup timing.

- TT (typical-typical): the nominal case. Both TT corners here run at 1.80 V.

- SS (slow-slow): slower than typical transistors. This is the pessimistic case for setup timing.

Lower supply voltage makes gates slower. The run has 6 FF corners (1.56 V to 1.95 V), 8 SS corners (1.28 V to 1.76 V) and 2 TT corners (1.80 V).

# 4. How to read a path report

- Startpoint and Endpoint: the flip-flop that launches the data and the flip-flop that captures it. The startpoint is core/CPU_is_addi_a3_reg in every corner, and the endpoint is a bit of the register file (core/CPU_Xreg_value_a4_reg[..][..]).

- Path Type: max: a setup (longest-path) check.

- Delay and Time columns: Delay is the time added by that one cell, and Time is the running total since the launching clock edge.

- ^ and v: the signal is rising (^) or falling (v) at that pin.

- Cell names: dfxtp_1 is a D flip-flop, nor2, nand2 and xor2 are basic gates, a21oi, o21ai, a21o and o22ai are compound gates, and fa is a full adder.

- clock network delay (ideal): the clock reaches every flip-flop at the same instant, because no clock tree has been built yet.

- Data arrival time: when the data actually reaches the endpoint.

- Data required time: the next clock edge (11.00 ns) minus the flip-flop setup time.

- Slack: required time minus arrival time. Positive slack is MET, negative slack is VIOLATED because the data arrives too late.

# 5. Timing summary and charts

## 5.1 Terminal output (Figure 1)

Figure 1: OpenSTA timing summary for all 16 corners.

Figure 1 is the terminal output printed at the end of the run. It has one row per corner and these columns:

- CORNER: the PVT corner that was analysed.

- MIN_SLACK: the smallest slack on the min-delay (hold) side. Every value is positive (0.187 ns to 1.830 ns), so there are no hold problems.

- MAX_SLACK: the worst slack on the max-delay (setup) side. It matches the slack at the bottom of each path report, for example 4.055 for FF_100C_1v65 and -62.563 for SS_n40C_1v28.

- WNS (worst negative slack): the most negative setup slack in that corner, or 0.000 when nothing fails.

- TNS (total negative slack): the sum of the negative slack over all failing endpoints. WNS shows how bad the single worst path is, and TNS shows how widespread the failure is. It is 0.000 for every passing corner.

- STATUS: MET when there are no setup violations, otherwise VIOLATED.

Reading the table: the six FF corners and the two TT corners are MET, and the eight SS corners are VIOLATED. The last lines confirm that 16 corners were processed. report_worst_slack gives -62.56 ns and report_tns gives -13957.59 ns, and both come from SS_n40C_1v28, the worst corner.

## 5.2 Results table (Figure 2)

Figure 2: Timing results for the 16 corners, collected in a spreadsheet.

Figure 2 holds the same numbers as Figure 1, arranged as one table for plotting. Each row is one corner and the columns are:

- Hold Slack: the MIN_SLACK column from Figure 1. It is positive in every row, so hold timing is met everywhere.

- Max Slack: the worst setup slack of the corner (the MAX_SLACK column).

- WNS and TNS: worst and total negative slack. Both are 0.000 when the corner passes, and both are shown in red when it fails.

- Status: worked out from WNS. A corner is MET when WNS is not negative and VIOLATED when it is negative.

The summary block under the table gives the headline results. 8 corners are MET and 8 are VIOLATED. The worst WNS is -62.563 ns, in corner SS_n40C_1v28. The total TNS across all corners is -36740.996 ns, which is the sum of the eight failing corners (for example -2599.514 + -892.705 + -13957.588 + ... + -367.928). This is different from the -13957.59 ns printed by report_tns in Figure 1, which is the TNS of the single worst corner only.

## 5.3 Charts (Figure 3)

Figure 3: WNS, TNS, setup slack range and hold slack across the 16 corners.

Figure 3 plots the columns of Figure 2. In the first three charts the horizontal axis is the PVT corner, in the order of the table (six FF corners, then eight SS corners, then two TT corners).

- Worst Negative Slack (WNS): the line is flat at 0 for the six FF corners, which pass. It drops at the SS corners, with -17.1 ns and -7.9 ns at 100 °C, then the -40 °C corners at -62.6 ns (1.28 V), -39.7 ns, -29.7 ns, -23.9 ns, -10.6 ns and -4.4 ns (1.76 V). It returns to 0 at the two TT corners. The deepest point of the chart is SS_n40C_1v28.

- Total Negative Slack (TNS): this chart has the same shape as WNS, with its lowest point at SS_n40C_1v28 (-13957.6 ns). The values are far larger than the WNS values because TNS adds up the failing slack of every endpoint. At SS_n40C_1v28 the worst path is late by 62.6 ns, but the total is -13957.6 ns, so many endpoints fail in that corner and not only one.

- Setup Slack Range (Min vs Max): the dark blue line is the Max Slack column (the worst setup path). It swings from +5.6 ns down to -62.6 ns. The light blue line is the Min Slack column, which holds the same values as the Hold Slack column, and it stays close to zero and above it in every corner. Together they show that setup timing changes a lot with the corner while the hold side barely moves.

- Hold Slack: the horizontal axis here is the corner number, 1 to 16 in table order (1 is FF_100C_1v65 and 16 is TT_100C_1v80). The line never goes below zero. It is smallest at corner 6 (FF_n40C_1v95, 0.187 ns) and highest at corner 9 (SS_n40C_1v28, 1.830 ns). Hold slack is largest in the slow corners because slow gates also lengthen the shortest paths, and smallest in the fast corners where the shortest paths are quickest. The lowest values are still positive, so no hold violation occurs in any corner.

# 6. Results for each corner

All times are in nanoseconds and the clock period is 11 ns. The required time is 11 ns minus the setup time of the capturing flip-flop, so it is slightly below 11 in every row.

## 6.1 Fast-process corners (FF)

| Corner | Worst endpoint (core/CPU_...) | Arrival (ns) | Required (ns) | Slack (ns) | Status |
| --- | --- | --- | --- | --- | --- |
| FF_100C_1v65 | Xreg_value_a4_reg[16][30] | 6.88 | 10.93 | 4.05 | MET |
| FF_100C_1v95 | Xreg_value_a4_reg[16][30] | 5.39 | 10.96 | 5.57 | MET |
| FF_n40C_1v56 | Xreg_value_a4_reg[16][31] | 8.64 | 10.92 | 2.28 | MET |
| FF_n40C_1v65 | Xreg_value_a4_reg[16][31] | 7.52 | 10.93 | 3.41 | MET |
| FF_n40C_1v76 | Xreg_value_a4_reg[1][31] | 6.52 | 10.95 | 4.43 | MET |
| FF_n40C_1v95 | Xreg_value_a4_reg[16][31] | 5.37 | 10.96 | 5.59 | MET |

All six FF corners meet timing. Arrival times run from 5.37 ns to 8.64 ns. A higher supply voltage gives a faster path (at -40 °C the arrival time falls from 8.64 ns at 1.56 V to 5.37 ns at 1.95 V). FF_n40C_1v56 has the least margin at +2.28 ns.

## 6.2 Typical-process corners (TT)

| Corner | Worst endpoint (core/CPU_...) | Arrival (ns) | Required (ns) | Slack (ns) | Status |
| --- | --- | --- | --- | --- | --- |
| TT_025C_1v80 | Xreg_value_a4_reg[27][30] | 8.90 | 10.84 | 1.94 | MET |
| TT_100C_1v80 | Xreg_value_a4_reg[27][30] | 8.81 | 10.90 | 2.09 | MET |

Both TT corners meet timing, with about 2 ns to spare. The results at 25 °C and 100 °C are very close (8.90 ns and 8.81 ns), so temperature has little effect at the nominal 1.80 V.

## 6.3 Slow-process corners (SS)

| Corner | Worst endpoint (core/CPU_...) | Arrival (ns) | Required (ns) | Slack (ns) | Status |
| --- | --- | --- | --- | --- | --- |
| SS_100C_1v40 | Xreg_value_a4_reg[27][30] | 27.54 | 10.42 | -17.12 | VIOLATED |
| SS_100C_1v60 | Xreg_value_a4_reg[27][30] | 18.51 | 10.64 | -7.87 | VIOLATED |
| SS_n40C_1v28 | Xreg_value_a4_reg[27][30] | 71.70 | 9.14 | -62.56 | VIOLATED |
| SS_n40C_1v35 | Xreg_value_a4_reg[27][30] | 49.47 | 9.77 | -39.70 | VIOLATED |
| SS_n40C_1v40 | Xreg_value_a4_reg[27][30] | 39.74 | 10.04 | -29.69 | VIOLATED |
| SS_n40C_1v44 | Xreg_value_a4_reg[27][30] | 34.10 | 10.20 | -23.91 | VIOLATED |
| SS_n40C_1v60 | Xreg_value_a4_reg[27][30] | 21.15 | 10.54 | -10.61 | VIOLATED |
| SS_n40C_1v76 | Xreg_value_a4_reg[27][30] | 15.07 | 10.69 | -4.38 | VIOLATED |

All eight SS corners fail setup. The data arrives between 15.07 ns and 71.70 ns after the launching edge, while the capture edge is only 11 ns away. The critical endpoint is the same register bit (Xreg_value_a4_reg[27][30]) in every SS corner. The slowest single cell on the path is the full-adder SUM output (core/U351/SUM), which takes 0.82 ns at SS_n40C_1v76 and 4.76 ns at SS_n40C_1v28. The setup time of the capturing flip-flop also grows, from 0.31 ns to 1.86 ns.

# 7. Walk-through of one path (FF_100C_1v65)

The critical path has almost the same shape in every corner, and only the delay of each cell changes. In the first report the data travels through these stages.

| Step | What happens | Cells | Time reached (ns) |
| --- | --- | --- | --- |
| 1 | Clock edge at 0.00 ns launches the data (clock-to-Q of core/CPU_is_addi_a3_reg). | dfxtp_1 | 0.24 |
| 2 | Control and decode logic. core/U9 (inv_2) is the slowest cell here at 0.42 ns. | nor2, nand2, inv_2, xor2 | 0.96 |
| 3 | A chain of 16 alternating AND-OR-INVERT cells, each adding about 0.09 ns to 0.19 ns. This pattern is typical of carry propagation. | a21oi, o21ai | 2.95 |
| 4 | A run of 11 full-adder cells mixed with a few other gates. This is the longest part of the path and adds about 3.6 ns. | fa_1, fa_2, a21o | 6.55 |
| 5 | Final gates that select the value written to the register, ending at the D pin of the capturing flip-flop. | nand2, o22ai | 6.88 |
| 6 | Capture edge at 11.00 ns minus 0.07 ns setup time gives a required time of 10.93 ns. | dfxtp_1 | Slack = 4.05 |

The long chain of full adders and carry gates suggests that the critical path runs through an adder. The report shows the cells but not their function, so this is an interpretation of the cell types.

# 8. Conclusions

- Hold timing is clean. Every corner has a positive MIN_SLACK, with a minimum of 0.187 ns.

- Setup fails in all SS corners. 8 of the 16 corners meet timing (all FF and TT) and 8 fail (all SS).

- The worst case is SS_n40C_1v28. WNS is -62.563 ns and TNS is -13957.588 ns (the total across all corners is -36740.996 ns). For that path to pass, the clock period would need to be at least about 73.6 ns (arrival 71.70 ns plus setup 1.86 ns).

- One path limits every corner. The same launch flop and the same adder-like chain set the worst path in every run.

- Voltage matters more than temperature. At -40 °C the SS slack improves from -62.56 ns at 1.28 V to -4.38 ns at 1.76 V. At the same voltage the colder corner is slower here (at 1.60 V the SS slack is -7.87 ns at 100 °C and -10.61 ns at -40 °C).

Before treating the SS failures as a design problem, decide whether the design is meant to run at supply voltages as low as 1.28 V to 1.40 V, and whether 11 ns is the intended clock period. If the failures are real, the usual options are a longer clock period, a faster adder structure, pipelining the path, or re-running synthesis with tighter constraints.

# 9. OpenSTA output for each corner

This section contains the worst setup path printed by OpenSTA for each of the 16 corners, in the order the script ran them. Each one is preceded by a one-line summary.

## 9.1 FF_100C_1v65

Fast process, 100 °C, 1.65 V. The worst path ends at core/CPU_Xreg_value_a4_reg[16][30]. Data arrives at 6.88 ns and is required by 10.93 ns, so the slack is 4.05 ns (timing met).

Running STA: FF_100C_1v65

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[16][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: FF_100C_1v65

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.24    0.24 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.09    0.34 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.08    0.42 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

0.42    0.83 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.13    0.96 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.19    1.16 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.09    1.25 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.15    1.40 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.09    1.49 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.15    1.64 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.09    1.73 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.15    1.89 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.09    1.98 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.15    2.13 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.09    2.22 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.15    2.37 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.09    2.46 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.15    2.61 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.09    2.71 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.15    2.86 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.09    2.95 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.14    3.09 v core/U96/X (sky130_fd_sc_hd__a21o_1)

0.28    3.36 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.14    3.50 v core/U94/X (sky130_fd_sc_hd__a21o_1)

0.26    3.76 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

0.27    4.03 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

0.27    4.31 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

0.28    4.59 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.04    4.62 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.05    4.68 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

0.26    4.94 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

0.27    5.21 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

0.27    5.48 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

0.29    5.77 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.14    5.91 v core/U81/X (sky130_fd_sc_hd__a21o_1)

0.26    6.17 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

0.38    6.55 ^ core/U351/SUM (sky130_fd_sc_hd__fa_1)

0.20    6.75 v core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.13    6.88 ^ core/U696/Y (sky130_fd_sc_hd__o22ai_1)

0.00    6.88 ^ core/CPU_Xreg_value_a4_reg[16][30]/D (sky130_fd_sc_hd__dfxtp_1)

6.88   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[16][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.07   10.93   library setup time

10.93   data required time

---------------------------------------------------------

10.93   data required time

-6.88   data arrival time

---------------------------------------------------------

4.05   slack (MET)

## 9.2 FF_100C_1v95

Fast process, 100 °C, 1.95 V. The worst path ends at core/CPU_Xreg_value_a4_reg[16][30]. Data arrives at 5.39 ns and is required by 10.96 ns, so the slack is 5.57 ns (timing met).

Running STA: FF_100C_1v95

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[16][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: FF_100C_1v95

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.20    0.20 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.07    0.27 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.07    0.33 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

0.35    0.68 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.09    0.77 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.15    0.92 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.07    0.99 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.12    1.11 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.07    1.18 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.12    1.30 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.07    1.36 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.12    1.49 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.07    1.55 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.12    1.67 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.07    1.74 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.12    1.86 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.07    1.93 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.12    2.05 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.07    2.12 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.12    2.24 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.07    2.31 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.11    2.42 v core/U96/X (sky130_fd_sc_hd__a21o_1)

0.22    2.63 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.11    2.74 v core/U94/X (sky130_fd_sc_hd__a21o_1)

0.20    2.95 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

0.21    3.16 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

0.21    3.37 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

0.22    3.58 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.03    3.61 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.04    3.66 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

0.21    3.86 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

0.21    4.07 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

0.21    4.28 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

0.22    4.51 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.11    4.62 v core/U81/X (sky130_fd_sc_hd__a21o_1)

0.20    4.82 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

0.30    5.12 ^ core/U351/SUM (sky130_fd_sc_hd__fa_1)

0.17    5.28 v core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.11    5.39 ^ core/U696/Y (sky130_fd_sc_hd__o22ai_1)

0.00    5.39 ^ core/CPU_Xreg_value_a4_reg[16][30]/D (sky130_fd_sc_hd__dfxtp_1)

5.39   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[16][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.04   10.96   library setup time

10.96   data required time

---------------------------------------------------------

10.96   data required time

-5.39   data arrival time

---------------------------------------------------------

5.57   slack (MET)

## 9.3 FF_n40C_1v56

Fast process, -40 °C, 1.56 V. The worst path ends at core/CPU_Xreg_value_a4_reg[16][31]. Data arrives at 8.64 ns and is required by 10.92 ns, so the slack is 2.28 ns (timing met).

Running STA: FF_n40C_1v56

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[16][31]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: FF_n40C_1v56

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.27    0.27 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.13    0.39 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.08    0.48 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

0.54    1.01 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.15    1.16 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.27    1.43 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.10    1.53 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.21    1.74 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.10    1.84 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.21    2.04 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.10    2.14 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.21    2.35 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.10    2.45 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.21    2.65 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.10    2.75 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.21    2.96 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.10    3.06 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.21    3.26 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.10    3.36 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.21    3.57 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.10    3.67 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.17    3.84 v core/U96/X (sky130_fd_sc_hd__a21o_1)

0.35    4.19 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.18    4.38 v core/U94/X (sky130_fd_sc_hd__a21o_1)

0.33    4.71 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

0.35    5.06 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

0.35    5.41 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

0.34    5.74 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.06    5.80 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.05    5.85 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

0.34    6.18 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

0.35    6.53 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

0.35    6.88 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

0.36    7.24 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.18    7.42 v core/U81/X (sky130_fd_sc_hd__a21o_1)

0.33    7.76 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

0.34    8.10 v core/U351/COUT (sky130_fd_sc_hd__fa_1)

0.22    8.32 ^ core/U617/X (sky130_fd_sc_hd__xor2_1)

0.16    8.48 v core/U618/Y (sky130_fd_sc_hd__nand2_2)

0.16    8.64 ^ core/U635/Y (sky130_fd_sc_hd__o22ai_1)

0.00    8.64 ^ core/CPU_Xreg_value_a4_reg[16][31]/D (sky130_fd_sc_hd__dfxtp_1)

8.64   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[16][31]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.08   10.92   library setup time

10.92   data required time

---------------------------------------------------------

10.92   data required time

-8.64   data arrival time

---------------------------------------------------------

2.28   slack (MET)

## 9.4 FF_n40C_1v65

Fast process, -40 °C, 1.65 V. The worst path ends at core/CPU_Xreg_value_a4_reg[16][31]. Data arrives at 7.52 ns and is required by 10.93 ns, so the slack is 3.41 ns (timing met).

Running STA: FF_n40C_1v65

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[16][31]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: FF_n40C_1v65

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.24    0.24 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.11    0.35 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.07    0.42 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

0.48    0.90 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.12    1.02 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.23    1.26 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.09    1.34 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.18    1.53 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.09    1.61 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.18    1.79 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.09    1.88 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.18    2.06 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.09    2.15 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.18    2.33 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.09    2.41 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.18    2.59 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.09    2.68 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.18    2.86 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.09    2.95 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.18    3.13 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.09    3.22 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.15    3.37 v core/U96/X (sky130_fd_sc_hd__a21o_1)

0.30    3.67 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.16    3.83 v core/U94/X (sky130_fd_sc_hd__a21o_1)

0.29    4.12 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

0.30    4.42 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

0.30    4.72 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

0.29    5.01 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.05    5.06 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.04    5.10 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

0.29    5.39 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

0.30    5.69 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

0.30    5.99 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

0.31    6.30 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.16    6.46 v core/U81/X (sky130_fd_sc_hd__a21o_1)

0.29    6.75 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

0.30    7.05 v core/U351/COUT (sky130_fd_sc_hd__fa_1)

0.19    7.24 ^ core/U617/X (sky130_fd_sc_hd__xor2_1)

0.14    7.38 v core/U618/Y (sky130_fd_sc_hd__nand2_2)

0.14    7.52 ^ core/U635/Y (sky130_fd_sc_hd__o22ai_1)

0.00    7.52 ^ core/CPU_Xreg_value_a4_reg[16][31]/D (sky130_fd_sc_hd__dfxtp_1)

7.52   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[16][31]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.07   10.93   library setup time

10.93   data required time

---------------------------------------------------------

10.93   data required time

-7.52   data arrival time

---------------------------------------------------------

3.41   slack (MET)

## 9.5 FF_n40C_1v76

Fast process, -40 °C, 1.76 V. The worst path ends at core/CPU_Xreg_value_a4_reg[1][31]. Data arrives at 6.52 ns and is required by 10.95 ns, so the slack is 4.43 ns (timing met).

Running STA: FF_n40C_1v76

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[1][31]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: FF_n40C_1v76

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.21    0.21 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.10    0.31 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.06    0.37 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

0.43    0.81 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.09    0.90 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.20    1.10 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.07    1.17 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.16    1.33 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.07    1.41 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.16    1.56 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.07    1.64 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.16    1.80 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.07    1.87 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.16    2.03 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.07    2.10 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.16    2.26 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.07    2.33 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.16    2.49 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.07    2.56 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.16    2.72 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.07    2.80 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.13    2.93 v core/U96/X (sky130_fd_sc_hd__a21o_1)

0.26    3.19 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.14    3.33 v core/U94/X (sky130_fd_sc_hd__a21o_1)

0.25    3.58 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

0.26    3.83 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

0.26    4.09 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

0.25    4.34 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.04    4.38 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.04    4.42 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

0.25    4.68 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

0.26    4.93 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

0.26    5.19 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

0.27    5.46 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.14    5.60 v core/U81/X (sky130_fd_sc_hd__a21o_1)

0.25    5.85 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

0.25    6.10 v core/U351/COUT (sky130_fd_sc_hd__fa_1)

0.17    6.27 ^ core/U617/X (sky130_fd_sc_hd__xor2_1)

0.12    6.39 v core/U618/Y (sky130_fd_sc_hd__nand2_2)

0.12    6.52 ^ core/U643/Y (sky130_fd_sc_hd__o22ai_1)

0.00    6.52 ^ core/CPU_Xreg_value_a4_reg[1][31]/D (sky130_fd_sc_hd__dfxtp_1)

6.52   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[1][31]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.05   10.95   library setup time

10.95   data required time

---------------------------------------------------------

10.95   data required time

-6.52   data arrival time

---------------------------------------------------------

4.43   slack (MET)

## 9.6 FF_n40C_1v95

Fast process, -40 °C, 1.95 V. The worst path ends at core/CPU_Xreg_value_a4_reg[16][31]. Data arrives at 5.37 ns and is required by 10.96 ns, so the slack is 5.59 ns (timing met).

Running STA: FF_n40C_1v95

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[16][31]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: FF_n40C_1v95

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.18    0.18 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.08    0.26 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.05    0.32 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

0.37    0.69 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.06    0.75 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.17    0.92 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.06    0.98 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.13    1.11 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.06    1.17 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.13    1.30 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.06    1.36 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.13    1.49 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.06    1.55 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.13    1.68 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.06    1.74 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.13    1.87 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.06    1.93 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.13    2.06 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.06    2.12 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.13    2.26 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.06    2.32 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.11    2.43 v core/U96/X (sky130_fd_sc_hd__a21o_1)

0.21    2.64 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.11    2.75 v core/U94/X (sky130_fd_sc_hd__a21o_1)

0.20    2.96 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

0.21    3.17 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

0.21    3.38 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

0.20    3.58 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.03    3.62 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.04    3.65 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

0.21    3.86 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

0.21    4.07 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

0.21    4.28 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

0.22    4.50 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.11    4.61 v core/U81/X (sky130_fd_sc_hd__a21o_1)

0.20    4.82 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

0.21    5.02 v core/U351/COUT (sky130_fd_sc_hd__fa_1)

0.14    5.17 ^ core/U617/X (sky130_fd_sc_hd__xor2_1)

0.10    5.27 v core/U618/Y (sky130_fd_sc_hd__nand2_2)

0.10    5.37 ^ core/U635/Y (sky130_fd_sc_hd__o22ai_1)

0.00    5.37 ^ core/CPU_Xreg_value_a4_reg[16][31]/D (sky130_fd_sc_hd__dfxtp_1)

5.37   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[16][31]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.04   10.96   library setup time

10.96   data required time

---------------------------------------------------------

10.96   data required time

-5.37   data arrival time

---------------------------------------------------------

5.59   slack (MET)

## 9.7 SS_100C_1v40

Slow process, 100 °C, 1.40 V. The worst path ends at core/CPU_Xreg_value_a4_reg[27][30]. Data arrives at 27.54 ns and is required by 10.42 ns, so the slack is -17.12 ns (timing violated).

Running STA: SS_100C_1v40

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[27][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: SS_100C_1v40

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.91    0.91 ^ core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.21    1.13 v core/U54/Y (sky130_fd_sc_hd__clkinv_1)

0.18    1.30 ^ core/U477/Y (sky130_fd_sc_hd__nand2_1)

0.71    2.01 v core/U51/Y (sky130_fd_sc_hd__nand2_2)

0.80    2.81 v core/U478/X (sky130_fd_sc_hd__a22o_1)

0.47    3.28 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.58    3.86 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.36    4.22 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.48    4.70 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.36    5.05 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.48    5.53 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.36    5.89 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.48    6.37 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.36    6.72 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.48    7.20 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.36    7.56 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.48    8.04 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.36    8.39 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.48    8.87 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.36    9.23 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.48    9.71 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.36   10.06 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.49   10.55 v core/U96/X (sky130_fd_sc_hd__a21o_1)

1.32   11.87 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.51   12.38 v core/U94/X (sky130_fd_sc_hd__a21o_1)

1.27   13.65 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

1.33   14.98 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

1.34   16.32 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

1.13   17.45 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.13   17.58 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.17   17.75 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

1.30   19.05 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

1.33   20.38 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

1.33   21.70 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

1.37   23.08 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.51   23.59 v core/U81/X (sky130_fd_sc_hd__a21o_1)

1.27   24.86 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

1.63   26.49 v core/U351/SUM (sky130_fd_sc_hd__fa_1)

0.66   27.15 ^ core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.39   27.54 v core/U684/Y (sky130_fd_sc_hd__o22ai_1)

0.00   27.54 v core/CPU_Xreg_value_a4_reg[27][30]/D (sky130_fd_sc_hd__dfxtp_1)

27.54   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[27][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.58   10.42   library setup time

10.42   data required time

---------------------------------------------------------

10.42   data required time

-27.54   data arrival time

---------------------------------------------------------

-17.12   slack (VIOLATED)

## 9.8 SS_100C_1v60

Slow process, 100 °C, 1.60 V. The worst path ends at core/CPU_Xreg_value_a4_reg[27][30]. Data arrives at 18.51 ns and is required by 10.64 ns, so the slack is -7.87 ns (timing violated).

Running STA: SS_100C_1v60

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[27][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: SS_100C_1v60

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.62    0.62 ^ core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.15    0.78 v core/U54/Y (sky130_fd_sc_hd__clkinv_1)

0.13    0.91 ^ core/U477/Y (sky130_fd_sc_hd__nand2_1)

0.51    1.42 v core/U51/Y (sky130_fd_sc_hd__nand2_2)

0.56    1.98 v core/U478/X (sky130_fd_sc_hd__a22o_1)

0.33    2.31 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.42    2.72 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.24    2.97 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.34    3.31 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.24    3.55 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.34    3.89 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.24    4.13 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.34    4.47 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.24    4.72 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.34    5.06 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.24    5.30 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.34    5.64 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.24    5.88 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.34    6.22 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.24    6.46 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.34    6.81 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.24    7.05 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.34    7.38 v core/U96/X (sky130_fd_sc_hd__a21o_1)

0.86    8.24 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.35    8.59 v core/U94/X (sky130_fd_sc_hd__a21o_1)

0.83    9.42 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

0.86   10.28 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

0.87   11.14 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

0.74   11.89 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.09   11.98 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.12   12.10 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

0.84   12.94 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

0.86   13.80 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

0.86   14.66 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

0.89   15.55 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.35   15.90 v core/U81/X (sky130_fd_sc_hd__a21o_1)

0.83   16.73 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

1.04   17.77 v core/U351/SUM (sky130_fd_sc_hd__fa_1)

0.48   18.25 ^ core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.26   18.51 v core/U684/Y (sky130_fd_sc_hd__o22ai_1)

0.00   18.51 v core/CPU_Xreg_value_a4_reg[27][30]/D (sky130_fd_sc_hd__dfxtp_1)

18.51   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[27][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.36   10.64   library setup time

10.64   data required time

---------------------------------------------------------

10.64   data required time

-18.51   data arrival time

---------------------------------------------------------

-7.87   slack (VIOLATED)

## 9.9 SS_n40C_1v28

Slow process, -40 °C, 1.28 V. The worst path ends at core/CPU_Xreg_value_a4_reg[27][30]. Data arrives at 71.70 ns and is required by 9.14 ns, so the slack is -62.56 ns (timing violated).

Running STA: SS_n40C_1v28

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[27][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: SS_n40C_1v28

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

1.50    1.50 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.71    2.20 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.56    2.76 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

2.67    5.43 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

1.71    7.14 v core/U479/X (sky130_fd_sc_hd__xor2_1)

1.42    8.57 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.76    9.33 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

1.20   10.53 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.76   11.29 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

1.20   12.49 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.76   13.25 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

1.20   14.45 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.76   15.21 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

1.20   16.41 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.76   17.17 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

1.20   18.38 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.76   19.14 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

1.20   20.34 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.76   21.10 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

1.20   22.30 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.76   23.06 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

1.13   24.19 v core/U96/X (sky130_fd_sc_hd__a21o_1)

3.70   27.89 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

1.32   29.21 v core/U94/X (sky130_fd_sc_hd__a21o_1)

3.62   32.83 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

3.78   36.61 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

3.80   40.42 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

3.39   43.81 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.40   44.21 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.21   44.43 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

3.63   48.06 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

3.78   51.84 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

3.78   55.62 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

3.86   59.48 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

1.32   60.80 v core/U81/X (sky130_fd_sc_hd__a21o_1)

3.62   64.42 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

4.76   69.19 v core/U351/SUM (sky130_fd_sc_hd__fa_1)

1.61   70.80 ^ core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.91   71.70 v core/U684/Y (sky130_fd_sc_hd__o22ai_1)

0.00   71.70 v core/CPU_Xreg_value_a4_reg[27][30]/D (sky130_fd_sc_hd__dfxtp_1)

71.70   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[27][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-1.86    9.14   library setup time

9.14   data required time

---------------------------------------------------------

9.14   data required time

-71.70   data arrival time

---------------------------------------------------------

-62.56   slack (VIOLATED)

## 9.10 SS_n40C_1v35

Slow process, -40 °C, 1.35 V. The worst path ends at core/CPU_Xreg_value_a4_reg[27][30]. Data arrives at 49.47 ns and is required by 9.77 ns, so the slack is -39.70 ns (timing violated).

Running STA: SS_n40C_1v35

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[27][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: SS_n40C_1v35

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

1.12    1.12 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.52    1.64 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.41    2.05 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

2.01    4.06 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

1.22    5.28 v core/U479/X (sky130_fd_sc_hd__xor2_1)

1.04    6.33 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.54    6.87 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.87    7.74 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.54    8.28 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.87    9.15 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.54    9.69 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.87   10.56 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.54   11.10 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.87   11.96 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.54   12.51 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.87   13.37 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.54   13.92 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.87   14.78 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.54   15.33 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.87   16.19 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.54   16.74 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.81   17.54 v core/U96/X (sky130_fd_sc_hd__a21o_1)

2.47   20.01 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.91   20.92 v core/U94/X (sky130_fd_sc_hd__a21o_1)

2.42   23.34 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

2.52   25.87 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

2.54   28.40 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

2.27   30.67 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.27   30.94 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.17   31.11 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

2.43   33.54 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

2.52   36.07 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

2.52   38.59 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

2.57   41.16 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.91   42.07 v core/U81/X (sky130_fd_sc_hd__a21o_1)

2.42   44.49 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

3.14   47.63 v core/U351/SUM (sky130_fd_sc_hd__fa_1)

1.20   48.83 ^ core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.64   49.47 v core/U684/Y (sky130_fd_sc_hd__o22ai_1)

0.00   49.47 v core/CPU_Xreg_value_a4_reg[27][30]/D (sky130_fd_sc_hd__dfxtp_1)

49.47   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[27][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-1.23    9.77   library setup time

9.77   data required time

---------------------------------------------------------

9.77   data required time

-49.47   data arrival time

---------------------------------------------------------

-39.70   slack (VIOLATED)

## 9.11 SS_n40C_1v40

Slow process, -40 °C, 1.40 V. The worst path ends at core/CPU_Xreg_value_a4_reg[27][30]. Data arrives at 39.74 ns and is required by 10.04 ns, so the slack is -29.69 ns (timing violated).

Running STA: SS_n40C_1v40

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[27][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: SS_n40C_1v40

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.94    0.94 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.43    1.38 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.34    1.72 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

1.70    3.42 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.99    4.40 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.87    5.27 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.44    5.71 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.71    6.43 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.44    6.87 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.71    7.58 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.44    8.02 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.71    8.74 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.44    9.18 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.71    9.89 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.44   10.33 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.71   11.05 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.44   11.49 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.71   12.20 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.44   12.64 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.71   13.36 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.44   13.80 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.66   14.46 v core/U96/X (sky130_fd_sc_hd__a21o_1)

1.95   16.41 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.74   17.14 v core/U94/X (sky130_fd_sc_hd__a21o_1)

1.91   19.05 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

1.98   21.04 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

2.00   23.03 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

1.79   24.82 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.22   25.04 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.14   25.18 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

1.92   27.10 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

1.98   29.09 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

1.98   31.07 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

2.02   33.10 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.74   33.83 v core/U81/X (sky130_fd_sc_hd__a21o_1)

1.91   35.74 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

2.46   38.20 v core/U351/SUM (sky130_fd_sc_hd__fa_1)

1.00   39.20 ^ core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.53   39.74 v core/U684/Y (sky130_fd_sc_hd__o22ai_1)

0.00   39.74 v core/CPU_Xreg_value_a4_reg[27][30]/D (sky130_fd_sc_hd__dfxtp_1)

39.74   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[27][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.96   10.04   library setup time

10.04   data required time

---------------------------------------------------------

10.04   data required time

-39.74   data arrival time

---------------------------------------------------------

-29.69   slack (VIOLATED)

## 9.12 SS_n40C_1v44

Slow process, -40 °C, 1.44 V. The worst path ends at core/CPU_Xreg_value_a4_reg[27][30]. Data arrives at 34.10 ns and is required by 10.20 ns, so the slack is -23.91 ns (timing violated).

Running STA: SS_n40C_1v44

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[27][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: SS_n40C_1v44

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.83    0.83 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.38    1.21 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.30    1.51 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

1.50    3.01 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.88    3.88 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.77    4.65 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.39    5.04 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.62    5.67 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.39    6.06 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.62    6.68 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.39    7.07 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.62    7.69 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.39    8.08 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.62    8.71 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.39    9.10 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.62    9.72 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.39   10.11 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.62   10.73 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.39   11.12 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.62   11.75 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.39   12.14 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.57   12.71 v core/U96/X (sky130_fd_sc_hd__a21o_1)

1.65   14.36 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.63   15.00 v core/U94/X (sky130_fd_sc_hd__a21o_1)

1.61   16.61 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

1.67   18.28 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

1.68   19.96 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

1.51   21.48 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.19   21.67 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.13   21.79 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

1.62   23.42 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

1.67   25.09 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

1.67   26.76 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

1.71   28.47 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.63   29.10 v core/U81/X (sky130_fd_sc_hd__a21o_1)

1.61   30.72 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

2.07   32.79 v core/U351/SUM (sky130_fd_sc_hd__fa_1)

0.87   33.65 ^ core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.45   34.10 v core/U684/Y (sky130_fd_sc_hd__o22ai_1)

0.00   34.10 v core/CPU_Xreg_value_a4_reg[27][30]/D (sky130_fd_sc_hd__dfxtp_1)

34.10   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[27][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.80   10.20   library setup time

10.20   data required time

---------------------------------------------------------

10.20   data required time

-34.10   data arrival time

---------------------------------------------------------

-23.91   slack (VIOLATED)

## 9.13 SS_n40C_1v60

Slow process, -40 °C, 1.60 V. The worst path ends at core/CPU_Xreg_value_a4_reg[27][30]. Data arrives at 21.15 ns and is required by 10.54 ns, so the slack is -10.61 ns (timing violated).

Running STA: SS_n40C_1v60

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[27][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: SS_n40C_1v60

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.57    0.57 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.25    0.82 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.20    1.02 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

1.04    2.06 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.53    2.58 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.52    3.10 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.24    3.35 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.41    3.76 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.24    4.00 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.41    4.41 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.24    4.66 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.41    5.06 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.24    5.31 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.41    5.72 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.24    5.96 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.41    6.37 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.24    6.62 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.41    7.03 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.24    7.27 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.41    7.68 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.25    7.93 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.37    8.30 v core/U96/X (sky130_fd_sc_hd__a21o_1)

0.98    9.28 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.40    9.68 v core/U94/X (sky130_fd_sc_hd__a21o_1)

0.96   10.64 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

0.99   11.63 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

1.00   12.63 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

0.91   13.53 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.12   13.65 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.09   13.74 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

0.96   14.71 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

0.99   15.70 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

0.99   16.69 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

1.02   17.71 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.40   18.11 v core/U81/X (sky130_fd_sc_hd__a21o_1)

0.96   19.06 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

1.21   20.27 v core/U351/SUM (sky130_fd_sc_hd__fa_1)

0.59   20.87 ^ core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.29   21.15 v core/U684/Y (sky130_fd_sc_hd__o22ai_1)

0.00   21.15 v core/CPU_Xreg_value_a4_reg[27][30]/D (sky130_fd_sc_hd__dfxtp_1)

21.15   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[27][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.46   10.54   library setup time

10.54   data required time

---------------------------------------------------------

10.54   data required time

-21.15   data arrival time

---------------------------------------------------------

-10.61   slack (VIOLATED)

## 9.14 SS_n40C_1v76

Slow process, -40 °C, 1.76 V. The worst path ends at core/CPU_Xreg_value_a4_reg[27][30]. Data arrives at 15.07 ns and is required by 10.69 ns, so the slack is -4.38 ns (timing violated).

Running STA: SS_n40C_1v76

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[27][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: SS_n40C_1v76

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.43    0.43 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.19    0.62 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.15    0.77 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

0.80    1.57 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.36    1.93 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.39    2.32 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.18    2.50 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.31    2.80 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.18    2.98 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.31    3.29 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.18    3.47 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.31    3.77 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.18    3.95 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.31    4.26 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.18    4.44 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.31    4.74 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.18    4.92 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.31    5.23 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.18    5.41 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.31    5.71 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.18    5.89 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.27    6.16 v core/U96/X (sky130_fd_sc_hd__a21o_1)

0.68    6.84 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.29    7.13 v core/U94/X (sky130_fd_sc_hd__a21o_1)

0.65    7.79 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

0.68    8.47 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

0.68    9.15 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

0.63    9.78 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.09    9.87 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.07    9.94 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

0.66   10.61 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

0.68   11.28 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

0.68   11.96 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

0.70   12.66 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.29   12.95 v core/U81/X (sky130_fd_sc_hd__a21o_1)

0.65   13.61 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

0.82   14.43 v core/U351/SUM (sky130_fd_sc_hd__fa_1)

0.45   14.88 ^ core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.19   15.07 v core/U684/Y (sky130_fd_sc_hd__o22ai_1)

0.00   15.07 v core/CPU_Xreg_value_a4_reg[27][30]/D (sky130_fd_sc_hd__dfxtp_1)

15.07   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[27][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.31   10.69   library setup time

10.69   data required time

---------------------------------------------------------

10.69   data required time

-15.07   data arrival time

---------------------------------------------------------

-4.38   slack (VIOLATED)

## 9.15 TT_025C_1v80

Typical process, 25 °C, 1.80 V. The worst path ends at core/CPU_Xreg_value_a4_reg[27][30]. Data arrives at 8.90 ns and is required by 10.84 ns, so the slack is 1.94 ns (timing met).

Running STA: TT_025C_1v80

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[27][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: TT_025C_1v80

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.28    0.28 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.12    0.40 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.10    0.50 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

0.51    1.01 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.18    1.19 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.25    1.43 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.11    1.54 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.19    1.74 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.11    1.84 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.19    2.04 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.11    2.15 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.19    2.34 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.11    2.45 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.19    2.64 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.11    2.75 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.19    2.94 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.11    3.05 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.19    3.24 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.11    3.35 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.19    3.54 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.11    3.65 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.17    3.82 v core/U96/X (sky130_fd_sc_hd__a21o_1)

0.39    4.21 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.18    4.39 v core/U94/X (sky130_fd_sc_hd__a21o_1)

0.37    4.75 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

0.38    5.13 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

0.38    5.52 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

0.36    5.87 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.05    5.93 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.06    5.98 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

0.37    6.35 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

0.38    6.73 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

0.38    7.11 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

0.40    7.51 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.18    7.69 v core/U81/X (sky130_fd_sc_hd__a21o_1)

0.37    8.06 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

0.45    8.51 v core/U351/SUM (sky130_fd_sc_hd__fa_1)

0.29    8.80 ^ core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.10    8.90 v core/U684/Y (sky130_fd_sc_hd__o22ai_1)

0.00    8.90 v core/CPU_Xreg_value_a4_reg[27][30]/D (sky130_fd_sc_hd__dfxtp_1)

8.90   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[27][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.16   10.84   library setup time

10.84   data required time

---------------------------------------------------------

10.84   data required time

-8.90   data arrival time

---------------------------------------------------------

1.94   slack (MET)

## 9.16 TT_100C_1v80

Typical process, 100 °C, 1.80 V. The worst path ends at core/CPU_Xreg_value_a4_reg[27][30]. Data arrives at 8.81 ns and is required by 10.90 ns, so the slack is 2.09 ns (timing met).

Running STA: TT_100C_1v80

Startpoint: core/CPU_is_addi_a3_reg

(rising edge-triggered flip-flop clocked by clk)

Endpoint: core/CPU_Xreg_value_a4_reg[27][30]

(rising edge-triggered flip-flop clocked by clk)

Path Group: clk

Path Type: max

Corner: TT_100C_1v80

Delay    Time   Description

---------------------------------------------------------

0.00    0.00   clock clk (rise edge)

0.00    0.00   clock network delay (ideal)

0.00    0.00 ^ core/CPU_is_addi_a3_reg/CLK (sky130_fd_sc_hd__dfxtp_1)

0.29    0.29 v core/CPU_is_addi_a3_reg/Q (sky130_fd_sc_hd__dfxtp_1)

0.11    0.40 ^ core/U474/Y (sky130_fd_sc_hd__nor2_1)

0.10    0.51 v core/U476/Y (sky130_fd_sc_hd__nand2_1)

0.48    0.99 ^ core/U9/Y (sky130_fd_sc_hd__inv_2)

0.19    1.18 v core/U479/X (sky130_fd_sc_hd__xor2_1)

0.23    1.41 ^ core/U569/Y (sky130_fd_sc_hd__a21oi_1)

0.12    1.53 v core/U571/Y (sky130_fd_sc_hd__o21ai_1)

0.18    1.71 ^ core/U574/Y (sky130_fd_sc_hd__a21oi_1)

0.11    1.83 v core/U576/Y (sky130_fd_sc_hd__o21ai_1)

0.18    2.01 ^ core/U581/Y (sky130_fd_sc_hd__a21oi_1)

0.11    2.12 v core/U583/Y (sky130_fd_sc_hd__o21ai_1)

0.18    2.31 ^ core/U587/Y (sky130_fd_sc_hd__a21oi_1)

0.11    2.42 v core/U589/Y (sky130_fd_sc_hd__o21ai_1)

0.18    2.61 ^ core/U344/Y (sky130_fd_sc_hd__a21oi_1)

0.11    2.72 v core/U213/Y (sky130_fd_sc_hd__o21ai_1)

0.18    2.90 ^ core/U343/Y (sky130_fd_sc_hd__a21oi_1)

0.11    3.02 v core/U210/Y (sky130_fd_sc_hd__o21ai_1)

0.18    3.20 ^ core/U342/Y (sky130_fd_sc_hd__a21oi_1)

0.11    3.32 v core/U209/Y (sky130_fd_sc_hd__o21ai_1)

0.18    3.50 ^ core/U341/Y (sky130_fd_sc_hd__a21oi_1)

0.11    3.62 v core/U215/Y (sky130_fd_sc_hd__o21ai_1)

0.17    3.79 v core/U96/X (sky130_fd_sc_hd__a21o_1)

0.38    4.16 v core/U942/COUT (sky130_fd_sc_hd__fa_1)

0.17    4.33 v core/U94/X (sky130_fd_sc_hd__a21o_1)

0.36    4.69 v core/U905/COUT (sky130_fd_sc_hd__fa_1)

0.37    5.06 v core/U887/COUT (sky130_fd_sc_hd__fa_1)

0.37    5.43 v core/U1370/COUT (sky130_fd_sc_hd__fa_1)

0.35    5.78 v core/U36/COUT (sky130_fd_sc_hd__fa_2)

0.05    5.83 ^ core/U87/Y (sky130_fd_sc_hd__clkinv_1)

0.06    5.89 v core/U613/Y (sky130_fd_sc_hd__o21ai_1)

0.36    6.26 v core/U352/COUT (sky130_fd_sc_hd__fa_1)

0.37    6.62 v core/U348/COUT (sky130_fd_sc_hd__fa_1)

0.37    6.99 v core/U35/COUT (sky130_fd_sc_hd__fa_1)

0.39    7.38 v core/U33/COUT (sky130_fd_sc_hd__fa_1)

0.17    7.55 v core/U81/X (sky130_fd_sc_hd__a21o_1)

0.36    7.91 v core/U347/COUT (sky130_fd_sc_hd__fa_1)

0.49    8.40 ^ core/U351/SUM (sky130_fd_sc_hd__fa_1)

0.25    8.65 v core/U3/Y (sky130_fd_sc_hd__nand2_1)

0.16    8.81 ^ core/U684/Y (sky130_fd_sc_hd__o22ai_1)

0.00    8.81 ^ core/CPU_Xreg_value_a4_reg[27][30]/D (sky130_fd_sc_hd__dfxtp_1)

8.81   data arrival time

11.00   11.00   clock clk (rise edge)

0.00   11.00   clock network delay (ideal)

0.00   11.00   clock reconvergence pessimism

11.00 ^ core/CPU_Xreg_value_a4_reg[27][30]/CLK (sky130_fd_sc_hd__dfxtp_1)

-0.10   10.90   library setup time

10.90   data required time

---------------------------------------------------------

10.90   data required time

-8.81   data arrival time

---------------------------------------------------------

2.09   slack (MET)